SAFETY DATA SHEET

## PRIMA QUICK PRIME

1.	Chemical Product and Company Identification

Product Name: PRIMA QUICK PRIME,

Universal, No-Rinse, Multi-Purpose Dual Cure Adhesive, Universal Bond Linker

Manufacturer: B.J.M. Laboratories Ltd.

Address: 12 Hassadna Street, Industrial Park, Or-Yehuda, 6022011, Israel.

Telephone: +972-74-7000111/ Fax: 972-3-7353020.

2.	Composition / Information of Hazardous Ingredients

| Hazardous Ingredients | CAS# | Hazard Identification & Risk Phrases |
| --- | --- | --- |
| 2-Hydroxyethylmethacrylate (HEMA) | 868-77-9 | Xi, H319/315-317 |
| Methacrylated phosphoric acid esters | 32435-46-4 | Xi, H319/315-317 |
| 4-Methacryloxyethyltrimellitic anhydride | 70293-55-9 | Xn; H302-319/315-317 |
| Acetone | 67-64-1 | H225, Xi, H302/319/EUH066/336 |
| Ethyl alcohol | 64-17-5 | H225, H302 |

- Additional information: For the wording of the listed risk phrases refer to section 16.

3.	Hazards Identification. Xi: Irritant   ; H: Flammable  .

Acute Health Hazards:  May cause eye, respiratory and skin irritation in high concentrations.

Signs and Symptoms of Exposure: High concentrations can cause eye and respiratory irritation. Prolonged skin exposure can cause dryness.

Primary Routes of Exposure: Inhalation, skin contact, eye contact.

Carcinogenicity: NTP – no, OSHA – no, IARC – no.

4.         First Aid Measures in Cause of Inhalation

If irritation occurs, take victim to open air.

In case of skin or eye contact flush with large quantities of water for 15 minutes.

5.	Fire-Fighting Measures

Flash Point: 18°C (64ºF Ethanol)

Extinguishing Media: Water spray, dry chemical.

Unusial Fire and Explosion Hazards: None.

6.	Accidental Release Measures

Clean-Up Procedure: Clean spill area with detergent and water.

7.         Handling and Storage

Handling: No special measures necessary if handled according to instructions.

Storage: Keep container tightly closed when not in use.

8.	Exposure Controls/ Personal Protection

Respiratory Protection: Avoid inhalation of vapors. No special protection

under normal use vapors.

Hand and Skin Protection: Avoid unnecessary skin contact.

9.	Physical and Chemical Properties

Appearance: Transparent liquid

Odour: Weak, ester like odour

Boling Point: N/A

Flash Point: 18°C (64ºF Ethanol)

Solubility in Water: Soluble

Vapor Density (Air = 1): N/E

Vapor Pressure (mmHg): N/E

10.        Stability and Reactivity

Stability: This material is stable.

Hazardous Reactions: None if handled according to directions

Hazardous Polymerization: None.

Conditions to Avoid: Heat.

Incompatibility: Reducing agents

11.        Toxicological Information

#### Irritation of Skin: Dryness and Irritation possible upon prolonged exposure.

Allergic Effects: Methacrylate sensitive individuals may exhibit an allergic

reaction.

12.         Ecological Information

Do not discharge into suvers or water supplies.

Environmental persistence and degradability: No data available.

Ecotoxicity: No data available.

- Disposal Considerations

May be disposed of in landfill or incinerator In accordance with local, state

and federal regulations.

- Transport Information

| 14.1 Transport at land | ADR | 3 | RID | 3 |
| --- | --- | --- | --- | --- |
|  | UN Number | 1170 | Kemler Number | --- |
|  | Packing Group | III |  |  |
|  | Proper shipping name | Ethanol Solution |  |  |
| 14.2 Transport at sea | ADNR | 3 | IMDG | 3 |
|  | UN Number | 1170 |  |  |
|  | EMS | F-E, S-D | MFAG |  |
|  | Packing Group | III |  |  |
|  | Proper shipping name | Ethanol Solution |  |  |
| 14.3 Air transport | ICAO / IATA-DGR | --- |  |  |
|  | UN Number | 1170 |  |  |
|  | Proper shipping name | Ethanol Solution |  |  |
|  | Subsidiary Risk | --- |  |  |
|  | Labels | Ethanol Solution |  |  |
|  | Packing Group | III |  |  |

U.S. DOT Shipping Name: Ethanol Solution

U.S. DOT Hazard Class: 3

Number: UN1170

U.S. Dot Packing Group: III

Other: Class 3 Flammable Liquids.

- Regulatory Information

This product requires classification according to the criteria of EC.

The product is a medical device according to the Council Directive 93/42/EEC and EU Regulation 2017/745 for Medical Devices.

| 15.1 UN Number | --- |
| --- | --- |
| 15.2 National regulations |  |
| 15.3 EU number |  |
| 15.4 Hazard symbols | Xi , H |
| 15.5 Hazard designation | Xi: Irritant . H: Flammable . Contains Ethyl Alcohol, HEMA, 4-Methacryloxyethyltrimellitic anhydride, Acetone. |
| 15.6 Risk phrases | ## R: 225, 302, 319, 335/315, 317, EUH066, 336 225 Highly flammable. 302 Harmful if swallowed. 319 Irritating to eyes. 335/315 Irritating to respiratory system and skin. 317 May cause sensitization by skin contact. EUH066 Repeated exposure may cause skin dryness or cracking. 336 Vapours may cause drowsiness and dizziness. |
| 15.7 Safety phrases | ## P: 102, 403, 210, 260, 305+351+338, 303+361+353, 262, 280 ## 102 Keep out of the reach of children. ## 403 Keep container in a well-ventilated place. ## 210 Keep away from sources of ignition - No smoking. ## 260 Do not breathe vapor. ## 305+351+338 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. ## 303+361+353 After contact with skin, wash immediately with plenty of water and soap. ## 262 Avoid contact with skin and eyes. 280 Wear suitable gloves and eye/face protection. |

U.S. – O.S.H.A. Status: This material is Xi: Irritant. F: Flammable under the criteria the U.S. Federal Hazard Communication Standard.

- Other Information

# These data are based on our present knowledge. However, they shall not constitute a guarantee for any specific product features and shall establish a legally valid contractual relationship.

##### Relevant H-phrases

225 Highly flammable.

302 Harmful if swallowed.

319 Irritating to eyes.

335/315 Irritating to respiratory system and skin.

317 May cause sensitisation by skin contact.

EUH066 Repeated exposure may cause skin dryness or cracking.

336 Vapours may cause drowsiness and dizziness.

Xn Harmful.

# Material should be maintained in original containers. Empty containers should not be re-used.

# AN SI / NFPA 704 DATA

Fire = 3

Health = 1

Reactivity = 1 Specific

This information is based on data considered accurate. However, no warranty is expressed or implied regarding the accuracy of these data, or the results to be obtained from use thereof.  In no event will the manufacturer or distributor be responsible for damages of any nature whatsoever resulting from the use of, or reliance upon this information.  General properties described herein are to be regarded as guides and are not guaranteed for all samples.